CSE421

COMPUTER NETWORKS

ASSIGNMENT 2

Name: Nuzhat Naushin

ID: 22201162

Section: 22

# Faculty: BIJS

## Answer to the question number 1

### (a) Calculations based on ipconfig output

#### (i) Broadcast Address

Given:
 IP Address: 19.253.99.49
 Subnet Mask: 255.254.0.0
 Prefix: /15

Network Address: 19.252.0.0
Broadcast Address: 19.253.255.255

(ii) Prefix Mask

The subnet mask 255.254.0.0 corresponds to /15

(iii) Second Usable IP Address

First usable IP address: 19.252.0.1
Second usable IP address: 19.252.0.2

(b) VLSM Sub-network Addresses

Starting Network: 19.252.0.0/15

| Network | Hosts Required | Required Size (2ⁿ) | Prefix | Network Address |
| --- | --- | --- | --- | --- |
| S_ONE | 1240 | 2048 | /21 | 19.252.0.0 |
| S_TWO | 510 | 512 | /23 | 19.252.8.0 |
| S_THREE | 3 + Routers | 8 | /29 | 19.252.10.0 |

## Hierarchical Network Tree for 19.252.0.0/15

19.252.0.0/15 (Parent Block)

│

├── S_ONE: 19.252.0.0/21 (1240 hosts)

│   └─ Range: 19.252.0.1 – 19.252.7.254

│

├── S_TWO: 19.252.8.0/22 (510 hosts + gateway)

│   └─ Range: 19.252.8.1 – 19.252.11.254

│

├── S_THREE: 19.252.12.0/29 (Router Multi-access, 6 hosts)

│   └─ Range: 19.252.12.1 – 19.252.12.6

│

└── WAN Links:

├── R1-R3: 19.252.12.8/30 (2 hosts)

│   └─ Usable IPs: 19.252.12.9 – 19.252.12.10

│

└── R3-R2: 19.252.12.12/30 (2 hosts)

└─ Usable IPs: 19.252.12.13 – 19.252.12.14

Answer to the question number 2

### (a) The administrator can use tracert or traceroute  to observe each hop in the path and identify where the connection fails.

(b) Creating a Server Problem

This is known as a Ping Flood or DoS attack. It is achieved by sending a continuous stream of ICMP Echo Request packets with large payloads at high frequency, for example: ping -t -l 65500 <target_IP>

(c) This is achieved using Port Forwarding (Static NAT).
The router maps external requests on port 28150 to the private IP address 192.168.10.10.

(d) DHCP Troubleshooting

A.The DHCP default-router is set to .12, but the HQ router interface Fa0/1 is configured with IP .15.
 The gateway IP provided to clients is incorrect.

B.The DNS server has a static IP 210.10.10.10, but this address was not excluded using ip dhcp excluded-address.
 The DHCP server may assign this IP to another device.

C. PC0 cannot obtain an IP address
 PC0 is located on a different network (193.10.10.0/24) separated by router BR.
 DHCP broadcasts do not cross routers unless a DHCP Relay Agent (ip helper-address) is configured on BR.

Answer to the question number 3

### (a) Dijkstra’s Algorithm

Using Dijkstra’s algorithm, the shortest paths from Node a to all other remote networks are computed by iteratively selecting the nearest unvisited node and updating the minimum path costs.

| Destination | Shortest Path | Cost |
| --- | --- | --- |
| c | a → c | 2 |
| b | a → c → b | 3 |
| d | a → c → b → d | 8 |
| e | a → c → b → d → e | 10 |
| z | a → c → b → d → z | 14 |

(b) Link State Neighbor Tracking

Routers use Hello packets sent at regular intervals to detect and maintain reachability of adjacent neighbors.

(c) Distance Vector Algorithms

#### (i) Periodic Updates

Periodic updates are generally inefficient because they consume bandwidth even when no topology changes occur and result in slow convergence.

#### (ii) Disadvantages

- Susceptible to routing loops (Count-to-Infinity problem)

- Slow convergence compared to Link State routing

Answer to the question number 4

### (a) Command Significance

#### (i) 0.0.0.0 0.0.0.0 s0/0/0

This is a Default Static Route.
 It directs all traffic for unknown destinations to interface s0/0/0.

#### (ii) Static Route with Administrative Distance 30

This is a Floating Static Route.
The higher Administrative Distance ensures the route remains inactive unless the primary route fails.

(b) Summary Route Problem

The subnets connected to Router A and Router C are discontiguous.
A summary route configured on Router B may overlap address ranges, causing traffic to be misrouted and resulting in data loss.

(c) Static Route Configuration

#### (i) Required Static Routes

For network 167.18.10.0/28:

On Router R1:
 ip route 167.18.10.0 255.255.255.240 <next-hop-IP>

On Router R2:
 ip route 167.18.10.0 255.255.255.240 <next-hop-IP>

(ii) Floating Static Route Configuration

To configure a floating static route:

- Add a higher Administrative Distance

- Use an alternate next-hop or exit interface

Example:
 ip route 167.18.10.0 255.255.255.240 <backup-next-hop> 5

This can be implemented on R1 or R2, depending on available alternate paths.

Answer to the question number 5

### (a) Mixed IPv4 and IPv6 Communication

IPv4 and IPv6 devices cannot communicate directly because they use different protocols.
 Solutions include:

- Dual Stack

- Tunneling

- NAT64

(b) Shortened IPv6 Address

Original Address:
FF10:00FF:0000:0000:AC19:0000:0000:E000

Shortened Form: FF10:FF::AC19:0:0:E000

(c) EUI-64 Address Generation

Given:
 MAC Address: F0-B2-F0-EA-DF-35
 Subnet ID: (1010)h

Steps:

- Invert the 7th bit (F0 → F2)

- Insert FFFE in the middle

Result:
 FE80:0000:0000:1010:F2B2:F0FF:FEEA:DF35

(d) DHCPv6 Concepts

#### (i) Stateful DHCPv6

False. A Router Advertisement with M-flag = 1 is required so that clients know to use DHCPv6.

#### (ii) Duplicate Address Detection (DAD)

DAD ensures that no two devices on the same link use the same IPv6 address.

Answer to the question number 6

### (a) ARP Protocol

#### (i) Cross-network ARP

ARP works only within the local network segment.
To communicate with a remote network, the host sends an ARP request for the default gateway’s MAC address.

(ii) MAC Addresses

ARP Frame (leaving Host A):
Source MAC: Host A
Destination MAC: FF:FF:FF:FF:FF:FF

Ping Frame (after ARP resolution):
Source MAC: Host A
Destination MAC: Default Gateway

(b) Frame Handling (Figure 06)

#### (i) Frame from Host A to Host E

Since switches have just powered on and MAC tables are empty, the frame is broadcast (flooded) out of all ports.

(ii) MAC Table of Switch S3

After Host A sends a frame to Host C:

| MAC Address | Port |
| --- | --- |
| Host A MAC | Port connected to A |
| Host C MAC | Port connected to C |
| Other MACs | Respective ports |

(c) MAC Address Analysis

Given MAC Address: E0:A9:B8:C7:D6:E5

#### (i) MAC Type

This is a Unicast MAC address because the least significant bit of the first octet is 0.

#### (ii) MAC Portability

MAC address portability means:

- It is burned into the NIC hardware

- It remains the same regardless of network location

- It is hardware-specific, not network-dependent

- The device retains the same MAC across different networks